Mobil Pegasus Special Product Description Mobil Pegasus Special is a high quality SAE 15W-40 gas engine oil primarily intended for the lubrication of crankcases and power cylinders of spark-ignited two- and four-cycle gas-fueled engines. It is ideally suited for engines subjected to a wide range of ambient temperatures during startup and operation. This oil is also recommended for the lubrication of the cylinders and crankcases of reciprocating compressors processing clean natural gas. Pegasus Special is formulated from high quality narrow-cut mineral base oil and a balanced ashless detergent additive system. It is designed to provide excellent protection of engine and compressor components while reducing combustion chamber deposits in all engines and port carboning in two-cycle engines. Pegasus Special exhibits a high level of chemical stability and resistance to oxidation and nitration to help provide long oil life and lower filter replacement costs. Mobil Pegasus Special has excellent anti-corrosion properties, which helps prevent corrosive wear in cylinders, valve areas and bearings resulting in longer engine life. The anti-wear properties helps minimise wear of rings, liners, and bearings. Features and Benefits Mobil Pegasus Special provides excellent low temperature performance providing faster lubricant supply to critical engine components particularly under cold start conditions. It provides good engine cleanliness while maintaining low wear rates which could result in improved engine performance, reduced maintenance costs and improved production capacity. Pegasus Special exhibits excellent chemical and oxidation stability resulting in long drain periods and reduced filter costs. Its excellent ashless detergency technology reduces deposit formation in combustion chambers and in two-cycle engine ports resulting in lower maintenance costs while improving engine performance and reducing fuel costs. Features; Advantages and Potential Benefits. Effective Low Temperature Performance; Improved cold-start lubrication. Excellent Anti-wear Properties; Helps lower engine component wear. Outstanding Oxidation and Chemical Stability; Cleaner Engines Extended drain interval potential Helps reduce oil filter costs Excellent resistance to oxidation and nitration. Ashless Formulation; Reduces port carboning in two-stroke cycle engines Controls combustion chamber deposit formation and Improves spark plug performance Helps Improve engine performance. Very Good Corrosion Resistance; Reduces valve guide wear in four stroke-cycle gas engines Protects bearings and internal components. Effective Dispersancy/Detergency; Cleaner engines Longer oil filter life Helps lower maintenance costs. Applications - Crankcases and power cylinders of spark-ignited two- and four-cycle gas engines - Engines subjected to remote startup under cold ambient conditions - Mainline applications where engines are shut down for long periods and must start and come on line quickly - Recommended for applications requiring ashless or very low ash formulations - Where port carboning of two-cycles gas engines is a concern - Reciprocating compressor cylinders compressing clean natural gas Typical Properties Mobil Pegasus Special. SAE Grade; 15W-40. Viscosity, ASTM D 445. cSt @ 40º C; 104. cSt @ 100º C; 13.7. Viscosity Index, ASTM D 2270; 132. Sulfated Ash, wt%, ASTM D 874; 0.07. Total Base #, mg KOH/g, ASTM D 2896; 2.4. Pour Point, ºC, ASTM D 97; -36. Flash Point, ºC, ASTM D 92; 236. Density 15.6º C, ASTM D 4952, kg/L; 0.88. Health and Safety Based on available information, this product is not expected to produce adverse effects on health when used for the intended application and the recommendations provided in the Material Safety Data Sheet (MSDS) are followed. MSDS's are available upon request through your sales contract office, or via the Internet. This product should not be used for purposes other than its intended use. If disposing of used product, take care to protect the environment. The Mobil logotype, the Pegasus design are trademarks of Exxon Mobil Corporation, or one of its subsidiaries. 1-2006.

|  | www.specmaslo.by Due to continual product research and development, the information contained herein is subject to change without notification. Typical Properties may vary slightly. This product data sheet is based on global product data sheets for use with products purchased in the U.S. © 2001 Exxon Mobil Corporation. All rights reserved. |  |
| --- | --- | --- |